CARLA Simulation Evaluation Report

Project: Vulnerable Road User (VRU) Safety Enhancement via Infrastructure-to-Vehicle (I2V) Communications

Environment: CARLA Autonomous Driving Simulator (Synchronous Mode, $\Delta t = 0.04s$)

1. Executive Summary

This evaluation compares the safety performance of an autonomous Ego Vehicle across two operating conditions when encountering pedestrian hazards at an intersection:

- Baseline Mode (Unassisted): Relies strictly on the vehicle's onboard front-facing semantic segmentation camera.

- V2X-Assisted Mode: Fuses onboard perception with real-time early warnings transmitted from an intelligent Roadside Unit (RSU) infrastructure camera.

The testing evaluates how early infrastructure line-of-sight offsets local occlusion or high-speed blind spots to mitigate collisions with VRUs.

2. System Architecture & Methodology

2.1 Perception Channels & Pedestrian Detection Logic

Both detection systems rely on Semantic Segmentation Cameras rather than standard RGB networks. This allows for mathematically precise object extraction by mapping environmental assets directly to specific color-coded ID tags.

[Raw Camera Stream] ---> [Isolate Red Channel (Labels)] ---> [Count Pixels == 4] ---> [Threshold Check]

- The Target Class (Pedestrians): In CARLA's semantic segmentation engine, pixel tags are encoded directly into the Red channel of the BGRA image array. The value 4 corresponds explicitly to Walkers (Pedestrians).

- Onboard Ego Detection Pipeline (semantic_callback_ego):

  - Array Parsing: Reshapes the raw buffer data into an $(H, W, 4)$ array.

  - Filtering: Targets the tag channel (labels = img_array[:, :, 2]).

  - Threshold: Triggers a detection state if the total sum of pedestrian pixels exceeds 150 pixels.

  - Ego Visualization: Multiplies the raw color tags by 10 (img_array[:, :, :3] * 10) to turn dark semantic tags into bright, visible color segments on screen.

- Infrastructure RSU Detection Pipeline (semantic_callback_rsu):

  - Vantage Sensitivity: Because the RSU is mounted high above the intersection, pedestrians appear smaller in its field of view. To compensate, its detection threshold is reduced to 40 pixels.

  - V2X Message Broadcasting: Upon triggering, it serializes a payload containing a timestamp and the estimated geographic location of the hazard:

$$\text{Estimated Location} = (x: 130.0, y: 141.0, z: 2.5)$$

2.2 Control & Braking Policies

[ Hazard Detected? ]

|

+----------+----------+

| YES                 | NO

v                     v

[ Brake Policy ]       [ Autopilot Cruise ]

- V2X: Smooth (0.5)    - Keep Active

- Local: Slam (1.0)

- Baseline Emergency Braking: Enforces a maximum braking force (brake=1.0) upon immediate local visual confirmation.

- V2X Early Braking: Enforces a predictive, smoother deceleration curve (brake=0.5) based on network broadcast frames, prioritizing passenger comfort and vehicle stability.

3. Experimental Setup & Scenarios

The evaluation runs across two distinct traffic density setups on the same map topology:

| Metric / Attribute | Scenario 1: Standard Crosswalk | Scenario 2: High-Density Crowd |
| --- | --- | --- |
| Ego Spawn Coordinate | $(120.0, 132.0, 2.0)$ | $(120.0, 132.0, 2.0)$ |
| VRU Spawn (Base) | $(130.0, 141.0, 2.5)$ | $(130.0, 141.0, 2.5)$ |
| Total Pedestrian Count | 6 Walkers | 60 Walkers |
| Pedestrian Velocity | $4.2 \pm 0.4 \text{ m/s}$ | $4.2 \pm 0.4 \text{ m/s}$ |
| Baseline Speed Modifier | High-Speed Profile (Late Detection Testing) | High-Speed Profile (Late Detection Testing) |

4. Key Performance Indicators (KPIs)

The simulation tracks and exports five distinct safety metrics to evaluate pipeline efficiency:

- Collision Rate (collision): Boolean indicating if the minimum distance dropped below the structural impact boundary ($< 1.95\text{m}$).

- Minimum Distance to Target (min_dist): The closest recorded distance vector between the vehicle bounding box and any active VRU agent.

- Brake Trigger Timestamp (brake_time): Chronological log in simulation seconds marking exactly when the vehicle changed its state out of cruise mode.

- Trigger Velocity (trigger_speed): Velocity profile entry speed ($\text{km/h}$) at the exact moment the braking command was issued.

- Network Transport Latency (latency_ms): Simulated communication lag applied exclusively to the V2X matrix (ranging randomly between $12.5\text{ms}$ and $18.2\text{ms}$).

5. Summary of System Limitations / Code Review Notes

[!CAUTION]

Critical Edge Case Identified in Control Loop Loopback

During the main execution block loop, the script explicitly overrides the safety braking logic by systematically re-enabling the Traffic Manager Autopilot and resetting flags on every single frame tick:

Python

# Lines 188-192

ego_vehicle.set_autopilot(True)

autopilot_disabled = False

- Impact: This creates a race condition where the vehicle applies brakes for a fraction of a second, only to be immediately put back into cruise mode by the Traffic Manager on the subsequent tick. For valid data extraction, these lines should be wrapped in a logical block that executes only after the hazard has cleared.